### Supporting information

S2 Table. Variables of RumiWatch® noseband sensors and 3D-accelerometers. All the activities were continuously recorded for 30’340 minutes divided in 1’440-minutes intervals (24h).

| Variable | Definition |
| --- | --- |
| Haltera |  |
| Eat time | Time spent intake, chewing, and swallowing of feed. Eating: >30 jaw movements/min, unsteady frequency |
| Eating chews | Overall number of eating jaw movements |
| Ruminate time | Time spent chewing and swallowing of ruminating boluses. Rumination: >30 jaw movements/min, minimum 3 min duration, steady frequency |
| Rumination chews | Number of ruminating chews per bolus |
| Rumination boluses | Number of ruminating boluses regurgitated |
| Other activity time | Time not engaged in eating, ruminating, or drinking activities during non-ingestive related activities. Other activity: characterized by <30 jaw movements/min, unsteady frequency. |
| Other chews Number of chews not attributable to any ruminating, feeding take, or drinking activity during non-ingestive related activities. |  |
| Pedometerb |  |
| Stand up | Event at which the pedometer angle changes its position from an angle >58° toward the vertical axis to an angle <58° toward the vertical axis |
| Lie down | Event at which the pedometer angle changes its position from an angle 58° toward the vertical axis for a duration of at least 50 s. |
| Lying time | Sum of the duration of all lying bouts within a given recording period. Lying bouts: period with the pedometer in a position exceeding an angle of 58° toward the vertical axis lasting >50 s. Interruption of this pedometer position for less than 50 s is identified and calculated as one stand-up and one lying-down event but not as a separate standing bout. The lying bout is rated as not interrupted. |
| Walking time | Sum of the duration of all walking bouts within a given recording period. Walking bouts: Period characterized by at least 3 consecutive strides in the same direction (forward or backward). The period between 2 strides must not exceed 4 s. Walking bouts are rated as separate if the time between 2 strides exceeds 10 s. |
| Standing time | Sum of the duration of all standing bouts within a given recording period. Standing bouts: periods during which the cow is in an upright position but not walking; temporary change of the pedometer angle exceeding 58° toward the vertical axis for less than 50 s is neither rated as lying-down and standing-up events nor as an additional lying bout. |
| Stride | One forward or backward movement of the limb within a walking bout. |

aZehner et al. (2017); bAlsaaod et al. (2015)

References

Alsaaod, M., J.J. Niederhauser, G. Beer, N. Zehner, G. Schuepbach-Regula, and A. Steiner. 2015. Development and validation of a novel pedometer algorithm to quantify extended characteristics of the locomotor behavior of dairy cows. J Dairy Res. 98:6236–6242.

Zehner, N., Umstätter, C., Niederhauser, J.J. and Schick, M., 2017. System specification and validation of a noseband pressure sensor for measurement of ruminating and eating behavior in stable-fed cows. Computers and Electronics in Agriculture, 136, pp.31-41.